HOME OWNER’S

INFORMATION PACK

for

Charleston,

Cove

(applicable to ground and first floor self-contained apartments)

www.scotia-homes.co.uk

Please read this document in conjunction with the NHBC booklet ‘Guide to your new home – A practical guide to looking after your new home’

Contents

Page

GENERAL MAINTENANCE and safety	4

OPERATING INSTRUCTIONS FOR GAS-FIRED CENTRAL HEATING AND HOT WAter SYSTEM	4

NPA (NATIONWIDE PROPERTY ASSISTANCE) EMERGENCY COVER	7

RADIATOR SAFETY PRECAUTIONS	8

HEATING AND HOT WATER INSTALLATION	9

GAS SYSTEM	9

HOT AND COLD WATER SERVICES	9

KITCHENS	15

EXTRACTOR FANS (dMEV SYSTEM)	23

VENTILATION AND AVOIDING CONDENSATION	24

COMMUNAL DIGITAL TELEVISION AND SATELLITE INSTALLATION	24

TELEPHONE INSTALLATION	28

WINDOWS	28

OPERATING INSTRUCTIONS FOR THE ELECTRICAL INSTALLATION	32

IF AN ELECTRICAL CIRCUIT FAILS	32

SMOKE, HEAT & CARBON MONOXIDE DETECTORS	33

EXTERNAL DOORS	33

INTERNAL DOORS	34

GARAGE DOORS (where applicable)	34

WALL TILING	35

SHOWER WALL PANELLING (where fitted)	35

INTERNAL DRAINAGE	35

FLOOR FINISHES	36

ROOF SPACE (Where applicable)	37

CONSTRUCTION OF WALLS, PARTITIONS, FLOORS & CEILINGS	38

FIXING TO WALLS, CEILINGS OR FLOORS – IMPORTANT NOTICE	39

external fixings	40

high performance keim masonry paint	40

K REND exterior features	40

MOCK CHIMNEYS	40

EFFLORESCENCE	40

EXTERNAL AREAS	41

CARBON dioxide MEMBRANE	42

ROOF rainwater sYSTEM	43

SURFACE AND RAIN WATER DRAINAGE CONSIDERATIONS	43

METERS	48

Utility suppliers	48

LOCAL AUTHORITY REFUSE AND RECYCLING COLLECTION	49

council tax	49

SCHEDULE OF TEST CERTIFICATES	50

SCHEDULE OF MATERIALS	51

NOTE:

The information contained in this document is for our standard house types and may not cover specific variations requested by you.

GENERAL MAINTENANCE and safety

Regular maintenance work is required for all homes to keep them at their best in the years to come and to ensure that they continue to be a safe home environment.  We recommend that you employ competent tradesmen/contractors to carry out the maintenance work, however if you decide to carry out maintenance work (or alteration works) on your home yourself, then there follows a list of some of the basic rules to bear in mind;

Always plan the job thoroughly in advance.

Consider any risks - is there adequate ventilation? Do you need any safety equipment? Can the job be done another way to make it safer? If you are in doubt then do not attempt the job yourself – seek advice from a professional or employ a skilled tradesman or contractor.

Check any materials you are going to use for any warnings or precautions and heed the material safety recommendations.

Always use the right tools for the job and use them in accordance with their instructions.

If you intend to work at height please be aware of the risks involved. Try to avoid working at height if at all possible but if you decide it is necessary then please make sure your ladder or stepladder is in good condition and securely held in place. There is a large amount of information and recommendations available on the subject of working at height on the internet or in most good public libraries – take the time to familiarise yourself with the risks and recommendations involved in working at height before carrying out the job.

If there is risk involved, try and avoid working alone.

Dispose of any surplus materials and waste according to the manufacturer’s instructions, adherence to Local Authority waste regulations, and consideration for the environment.

Always keep a well stocked first aid kit.

Please also refer to your NHBC ‘Guide to Your New Home’ for more information on maintenance.

OPERATING INSTRUCTIONS FOR GAS-FIRED CENTRAL HEATING AND HOT WAter SYSTEM

Introduction

Your home has been fitted with a gas-fired heating system serving radiators and providing a domestic hot water supply.

The gas–fired boiler is located in the utility or utility cupboard and you will find the operating and maintenance instructions for the boiler in your handover pack.

If, after referring to the user information on boiler controls in the boiler instruction manual, you are unable to find the answers to any boiler problems and the problem is an emergency which has arisen during the first 24 months after your legal date of entry, then please contact NPA (please refer to the section on NPA below for more details). If the fault is not an emergency (as described in the NPA cover summary) then please contact Scotia during normal office hours.

You are responsible for the annual maintenance and servicing of the boiler, this should be arranged through any reputable, Gas Safe registered, plumbing and heating contractor.

Heating and Domestic Hot Water Controls

The system has the following controls:-

- Boiler isolating switch.

- Programmable Room Thermostat

- Thermostatic radiator valves to radiators (except on the by pass radiator/s)

1. Boiler Isolating Switch

This switch will typically be found on the wall next to the boiler in the ground floor utility or utility boiler cupboard.

This switch is to isolate the electrical supply to the boiler and should be left on at all times. Only use this switch if a fault develops on the boiler.

2. Programmable Room Thermostat

Your home has a programmable room thermostat located on the wall, typically in the hall or in the kitchen/living room area or in the boiler cupboard and it may be linked to a remote sensor typically located in the hall (locations depends on your flat type).

Examples of typical programmable room thermostats (the picture on the left shows a Danfoss TP7001 programmable room thermostat and the picture on the right shows a Danfoss TP9000 programmable room thermostat - the model installed may vary depending on your flat type);

Drawing of a typical remote temperature sensor (linked to the programmable room thermostat);

The programmable room thermostat controls the boiler, telling it when you require central heating and hot water. The programmer has the facility to give several on/off times. Temperatures can also be selected for each on/off time. When the heating is selected on - the system will operate until the set temperature is achieved. Note that when the programmable room thermostat is linked to a remote temperature sensor then the temperature is measured at the remote temperature sensor location (not at the programmable room thermostat location) . Where fitted, the remote temperature sensor should not be covered or otherwise obstructed as this may impair its ability to accurately measure the temperature.

When the system is selected off, the boiler will not operate unless the temperature in the hallway drops below the minimum setback temperature selected. You can select your minimum desired temperature, normally this can be set between 12 and 16°C. Please refer to the manufacturer’s instructions in your handover pack for the programmable room thermostat for further instructions.

3. Thermostatic Radiator Valves

Thermostatic Radiator Valves (TRV’s) are fitted for comfort control i.e. 1 – low level heat, 5 – maximum level heat. They are fitted to all radiators except radiators located where there is a room thermostat sensor. They are essential to the full efficiency of your heating system and allow you to lower temperatures in unoccupied rooms thus reducing heating costs. TRV’s sense room temperature changes in individual rooms and adjust the flow of heated water through the radiators to maintain the desired temperature. Depending on level of comfort required, 2 – 3 should be selected. Please refer to the manufacturer’s instructions in your Handover Pack for full details.

Picture showing a typical Thermostatic radiator valve, Note- the type fitted in your home may vary.

Central Heating

Should your central heating or hot water fail to work, please ensure that all of the procedures laid out in the boiler manufacturer's literature are followed. Failure to do this may result in a charge being made for an unnecessary call out.

Bleeding of Radiators

This should not be required with a sealed system. However, radiators feeling warm at the bottom but cold at the top would indicate air in the radiator. There are airing points normally at the top of the radiator. An air-bleeding key (available at any hardware store) can be used to allow trapped air to escape.

1. Keep your heating system on low so it remains pressurised, but be aware that the water in your radiators will be hot, so do take extra care.

2. Attach a radiator key to the bleed valve, usually located at the top and to one end of the radiator, and slowly begin to turn anti-clockwise.

3. Keep turning until you hear a slight hiss of air. When water begins to escape instead of air, you know it’s time to close the valve as all of the trapped air has now escaped. Hot water may spurt out quite quickly so do protect yourself and the area around.

4. Tighten up the valve by turning the key clockwise.

5. Check the pressure gauge on the boiler, if it is below that recommended in the manufacturer's instructions, then it will be necessary to top up the system. See enclosed boiler user guide for full instructions.

Throughout this process you should always place a cloth beneath the valve to catch any water that might leak from it, which could discolour floor coverings due to the additives within the heating system

You may find the following video useful as an example of how to bleed a radiator:

https://www.youtube.com/watch?v=UqK53MPioHI

NPA (NATIONWIDE PROPERTY ASSISTANCE) EMERGENCY COVER

Your new home is covered under NPA’s Home Emergency Assistance Cover for a period of 24 months from your legal date of entry.

Details of the cover will be provided direct to you by NPA and the following notes provide some outline points for your information.

The NPA cover includes items such as blocked or burst pipes (where they occur within your apartment) and damaged window and door locks to your flat where there is a risk to security. Some examples of what is/ is not covered are shown in the list below- please note that some of these examples may not apply to your own property.

In an emergency situation (as defined in the Cover Summary which will be provided to you direct by NPA) where any of the covered items are affected you should contact NPA directly instead of Scotia.

At the time of writing, the emergency contact number for NPA is 0345 1552374.

Any non-emergency problem occurring inside your apartment (and occurring within 24 months of your legal date of entry and which is not classed as an emergency by NPA) should be reported to the Scotia customer care department during normal office hours.

Examples of what is covered/ not covered by NPA;

| Examples of what is covered | Examples of what is not covered |
| --- | --- |
| Internal Plumbing & Drainage (inside your apartment) Repairs to your internal plumbing and drains including where there is a loss of water to your property, blocked drainage and leaks (inside your apartment) Clearing blocked toilets and waste pipes (inside your apartment) | Showers including the shower unit, controls, outlet or shower head Replacement of water tanks/radiators, thermostatic radiator valves, hot water cylinders and sanitary ware |
| Clearing total blockages to your drainage pipes (inside your apartment) | Shared drains External guttering, rainwater downpipes, rainwater drains and soakaways |
| Electrical Emergency & Breakdown An emergency caused by, or breakdown of, the domestic electrical wiring supply system, including permanent damage caused by a power cut | All non-permanent wiring/electrics, e.g. kettles, fairy lights or any other and appliances with plugs Shower unit or immersion heater unit |
| Security Making your property secure in the event of broken locks for external windows and doors that are your responsibility | Doors (and windows) which do not secure your property, such as internal porch doors, internal doors and internal conservatory doors Failure of breakdown of the external locking mechanisms to doors or windows for outbuildings or a garage |

RADIATOR SAFETY PRECAUTIONS

Myson Premier HE roundtop radiators have been installed in your property. Should radiator chip or paint damage occur then touch up paint (RAL 9016) can be purchased from the Myson sales office on 0845 402 3434.

Radiators are hot when in use and as such users should ensure that those who may come into close proximity to hot radiators are aware of the risks of burns on prolonged contact.

Where necessary, users should take steps to minimise the risks of burns from hot radiators (for example where there are very young children in the room). Where applicable, consideration should be given to placing guards in front of the radiators or reducing the temperature of individual radiators by turning the thermostatic radiator valve to a low setting.

Radiators are heavy items and are securely fastened to the wall on installation, with appropriate fasteners to secure the radiator bracket and suit the construction of the wall.

Radiators should not be used for the mounting of clothes airers, cat beds or other such fixtures. The mounting brackets of the radiator are designed to support the weight of the radiator itself and water contents, allowing for an adequate safety margin, and additional weight may compromise this margin and cause risk of failure, leaks and potential hot water burns.

Decorative covers (such as the decorative perforated MDF or timber covers that you can purchase from DIY stores) will significantly reduce the output of a radiator and thermostatic radiator valves should not be fitted inside these radiator covers (as this will stop the valves from working efficiently). These covers are not recommended as they will, by consequence, impede an individual room’s heat requirement, which your new heating system has been carefully designed to provide. The only exception to this would be the comments above regarding safety of young children. If you do need to fit a radiator cover for this purpose then you should use one of the metal mesh type covers, similar to a fire guard, which will not impede the flow of heat from your radiator into the room.

Appropriate facilities are required by Building Regulations for internal and/or external drying facilities and these are provided (please see “Ventilation and avoiding condensation”). Radiator-mounted airers and other devices may lead to excessive internal moisture and any chips/damage caused to the radiator itself may compromise the protective coating and potentially lead to corrosion/failure, which may not be covered by warranties.

Note regarding curtains – Heavy curtains or lined curtains drawn over the windows are an effective way of reducing any heat loss through the windows during the autumn and winter months. The sooner you draw the curtains in the evening the more heat you will save. However, please note that if there is a radiator located below the window, and the curtains are too long and cover the radiator then much of the heat generated by the radiators will not find its way into the room and will instead be wasted out the window. Long curtains covering the thermostatic radiator valves will also interfere with the operation of the valves.

HEATING AND HOT WATER INSTALLATION

Heating and hot water are provided by an Alpha InTec 34C wall mounted, high efficiency boiler with a gas saver flue gas heat recovery unit.

A 25 or 50 litre thermal store may also have been installed (the installation of a thermal store depends on the flat type).

Please note that to obtain the maximum performance and efficiency benefits the gas saver flue (and thermal store where fitted) must be switched on. Turning off the gas saver flue will mean you lose efficiency benefits. Turning off the thermal store (where fitted) will mean you lose efficiency benefits and the available volume of hot water will be significantly reduced.

A copy of the user manual, installation & service instructions, inspection, commissioning and service record logbooks for the system are enclosed  with your handover pack.

An annual gas service of the boiler and inspection/maintenance of the associated equipment, in accordance with the above instructions, is required to be carried out by Gas Safe registered personnel.  Failure to carry this out will invalidate the manufacturer’s and NHBC warranties.

GAS SYSTEM

Never obstruct gas boiler flue outlets or any ventilation, if provided, to the boiler. Never tamper with the gas installation or equipment.

Any alterations to the gas supply pipework or work in construction with any gas appliance should only be carried out by GasSafe registered personnel.

If you suspect a gas leak:

1.	Extinguish all naked flames.

2. 	Do not use any electrical switches or appliances

3.	Turn off the gas at the meter.

4.	Open all doors and windows.

5.	Call the National Gas Emergency Service on its emergency number which is in the telephone directory under ‘GAS, Gas Emergency’. There is no call-out charge. The current emergency number at date of preparation of this document is 0800 111999. This service operates 24 hours a day and 365 days a year.

HOT AND COLD WATER SERVICES

Mains Cold Water Service

The stopcock for the incoming cold water service is normally located under the kitchen or utility sink.

There is also a shut off valve located in an underground box in the public footpath (usually to the front of the apartments). This outside shut off valve is for use by Scottish Water only.

Make sure that you know where all mains water stopcocks are located so that you can turn them off quickly in an emergency.

Domestic Hot Water

The Alpha Intec 34C boiler produces domestic hot water in an energy efficient manner and the operation of this is fully explained in the enclosed Instruction Manuals. Please also refer to the notes in the Heating and Hot Water Installation section above.

External Water Tap (if fitted)

Where there is a risk of severe frost, water supply to tap should be isolated, pipe work drained, and the tap should be left in the open position.

We also recommend fitting an insulating external tap cover to external taps during winter months – these covers are available from any good gardening or DIY store.

Sanitary Ware/Taps

Introductory notes;

Sanitary ware and taps should be cleaned in accordance with the manufacturer's instructions, copies of which are enclosed in your Handover Pack or are available from the manufacturer’s web site. The following are some points to remember when cleaning and maintaining your sanitary ware and taps.

Note that the information in this section covers standard sanitary ware and fittings being installed at the date of preparation of this document- if you have asked for different fittings as a client choice then the following notes may not be applicable.

Baths (where applicable to the flat type);

The bath should always be cleaned immediately after use, preferably while the water is running out and the bath is still warm. Hot soapy water with a cloth should be used and the bath wiped dry.

Never allow solvents such as dry cleaning agents or paint strippers to come into contact with an acrylic bath.

Never use gritty or abrasive cleaners on the sanitary ware which can cause scratches and other damage.

The manufacturer of the bath recommends the use of an anti-slip mat when a shower is installed over the bath.

Bath pop-up wastes (where fitted);

The following is an extract from the bath pop-up waste manufacturer’s instructions;

WCs

Please note that WC cistern valves should be checked periodically and maintained as described in the manufacturer’s instructions contained in your hand over pack.

We recommend that you contact a plumber if you suspect that a WC valve has developed a fault.

Wash hand basin taps and bath taps;

The following are extracts from the Bristan manufacturer’s aftercare instructions for the wash hand basin and bath taps fitted as standard;

Bristan Prism basin and bath mixer taps (cleaning and care);

Note: if your tap begins to drip then maintenance is required - please refer to the instructions contained in your hand over pack (extracts of which are also copied below). We recommend that all the maintenance work on your taps is carried out by a qualified plumber.

Bristan Prism bath filler parts list and maintenance;

Bristan Prism basin mixer tap parts list and maintenance;

Bristan Prism recessed thermostatic dual control shower valve (where applicable);

The following are extracts from the manufacturer’s instructions for your dual control shower valve (if fitted) (Bristan PM2 SHCVO C);

Bristan Prism shower valve general cleaning instructions;

Bristan Prism shower valve cartridge maintenance;

Note- we recommend that all maintenance work on your shower valve is carried out by a qualified and experienced plumber.

Bristan Prism dual shower valve operating instructions;

Bristan Prism dual control shower valve – adjusting the temperature;

The shower has been factory set to 42 degrees centigrade (this is a safety feature). It is possible to adjust the temperature setting and the ‘installation instructions and user guide’ for the Bristan thermostatic recessed dual control shower valve contained in your hand over pack provides details. Please note we recommend that any adjustments to the temperature setting must be carried out by a qualified and experienced plumber.

Kitchen sink and tap

Please refer to the kitchens section of this Home Owner’s Information Pack for cleaning and maintenance instructions for the kitchen sink and kitchen mixer tap.

Thermostatic mixing valve (where there is a bath installed)

A thermostatic mixing valve to limit the hot water temperature at the bath tap to a maximum of 46°C is fitted below the bath.

The thermostatic mixing valve manufacturer’s instructions recommended that you arrange for initial temperature checks to be carried out six weeks after occupation of the property, and then for an annual check to be carried out. This is to test the water temperature from the tap to make sure that the mixing valve is operating properly and to ensure the maximum limit of 46°C is not exceeded.

Hot Water Temperature (kitchen sinks and wash hand basins)

The hot water from your kitchen sink taps and wash hand basins can be very hot depending upon the boiler settings. The hot water can initially have a low temperature as cooler water sitting in the pipes is discharged but can then become hot suddenly. Appropriate care should be taken to avoid risks of scalding.

KITCHENS

Appliances, kitchen and utility sinks, taps and units/worktops

Refer to the manufacturer’s instructions for operating and cleaning of kitchen appliances, sinks, units and worktops. These are enclosed in your Handover Pack (and some typical extracts have also been included below for your convenience).

Any appliance fitted within the property such as oven, hob, extractor hood, etc. will be covered by the manufacturer’s normal warranty. Smeg appliances only – these are covered by a manufacturer’s 2 year warranty which is pre-registered, do not re-register this warranty as this may cause the warranty inadvertently to revert to 1 year. In the event of a fault, a Smeg contact information sheet is included within the handover material, post code and plot handover date must be quoted for reference.

Note: The eye-level fridge in the AT10 apartment type may be a Bosch model, the same 2 year warranty terms apply, please refer to (manufacturer) handover material for warranty contact information.

Handles:

Kitchen designs are planned with the minimum corner post size as per the manufacturer’s recommendation to avoid a handle clash. Depending on the handle choice, the handles may still touch, which is perfectly acceptable, providing an opening of 85° can still be achieved. As with any door, opening and closing should be done carefully in order to avoid over-stressing the hinges or clash with another surface. After-market hinge restrictors can be obtained from kitchen suppliers in order to limit this angle to the maximum recommended, as above.

Notes on appliances;

Cooker hoods – Please note that the kitchen cooker hood has been installed for use in the recirculation mode* and a charcoal filter has been fitted.

*Note- depending on the manufacturer of your cooker hood this may be referred to as filter version or filter mode instead of recirculation mode.

Generally, when charcoal filters are fitted to a cooker hood they normally require changing after every three/four months or more frequently if used more than 3 hours a day. Please refer to the manufacturer’s instructions contained in your hand over pack for specific advice for the make/ model of cooker hood applicable to your property. Replacement charcoal filters are available from on-line retailers. Charcoal filters are not washable or cleanable.

Also, please note that the cooker hood grease filter(s) and the cooker hood should be cleaned at least once per month or in line with the recommendations for the make/model of cooker hood installed in your property. Before any maintenance or cleaning is carried out the cooker hood electricity supply should be switched off (and after cleaning ensure that all parts are dry before turning the electricity supply back on).

As with all cooker hoods, there can be a fire hazard if the grease and charcoal filters are not cleaned and/or replaced (and if the cooker hood body is not cleaned) as recommended in the manufacturer’s instructions. Flaming of foods beneath the hood should be avoided and frying must be done with care in order to make sure the oil does not overheat and burst into flames. Please refer to the instruction manual for your cooker hood which is contained in your Handover Pack for more details, and for other important operating and maintenance information for your cooker hood.

Connecting appliances – (where applicable)

Electrical connections for cookers and other electrical appliances requiring wiring work should be made by a qualified electrician using the pre-fitted wiring and fittings installed for this purpose.

When fitting a dishwasher or washing machine, please ensure the blanked end of the waste pipe tee piece has been removed. Note – this is not applicable where a ‘standing waste’ pipe has been provided. Please also ensure that the water supplies and wastes are securely connected to the pipework.

Please also note that, where Scotia have not installed a washing machine but have left a space for one with a cold water supply adjacent to it, then the home owner is responsible for removal of the cap that has been fitted to the cold water washing machine valve – please ensure you remove this cap before making the water connection to your washing machine. Please also ensure that the appliance water supply and waste pipe is securely connected to the apartment’s pipework before turning the appliance on.

Kitchen sink after-care instructions

The following is an extract from the kitchen sink (and utility sink, where applicable) manufacturer’s after-care instructions;

Kitchen sink mixer (tap) – maintenance

The following is an extract from a typical kitchen sink tap instructions- a Bristan Ruby Kitchen Sink mixer (please refer to your handover pack for further details or for instructions for other types of kitchen sink taps or for instructions for your utility sink, if applicable);

‘Parts list’ and maintenance notes;

Notes on isolation valves;

Isolation valves may have been fitted to your kitchen sink tap - these valves, where fitted, allow the water to be turned off to the tap to allow maintenance work to be carried out on the tap (instead of shutting off the water to the whole apartment). The two diagrams shown below show the isolating valves in the closed and open positions.

Kitchen units and worktops care notes;

Extracts from typical kitchen manufacturer’s after care instructions for kitchen units (note that these are typical instructions and may not cover all aspects of the kitchen fitted in your home- refer to the handover pack for more information on cleaning and maintaining your kitchen);

Worktops;

General notes on kitchen units;

Glass splash back to hobs- care and maintenance notes;

A glass ‘splash back’ manufactured by Intaglio Glass and Design may have been fitted behind your kitchen hob. The following is an extract from the manufacturer’s iCoat Colour Care and Maintenance instructions for this splash back;

EXTRACTOR FANS (dMEV SYSTEM)

Greenwood Airvac Unity CV2GIP mechanical extract fans have been fitted in your home. These are continuously running single point dMEV (decentralised Mechanical Extract Ventilation) fans with GIP (Guaranteed Installed Performance). The fans run continuously at a low (extremely quiet) speed and are automatically ‘boosted’ to a higher speed when required (the fans boost automatically using their humidity sensor). It is essential that the fans remain in operation at all times (unless switched off for maintenance) to maintain good air quality.

Pictures of the Greenwood Airvac CV2GIP fan

This is an energy efficient fan designed to provide an economical ventilation solution to the modern home. It utilises SMART Technology to control humidity and boost run on times to minimise the periods of time when it is running at its highest speed, minimising nuisance running noise and unnecessary energy wastage and heat loss typically associated with ‘traditional’ extract fans.

More information on these ventilation fans can be found in the User/Homeowner Guide contained in your handover pack. Please read these instructions carefully to ensure their continuing smooth operation.

VENTILATION AND AVOIDING CONDENSATION

Condensation will be a problem in all new houses if adequate background heating and ventilation is not used. All new homes need ‘running-in’ and we recommend that you read carefully the section within the NHBC booklet: GUIDE TO YOUR NEW HOME, ‘Condensation’.

Most windows are fitted with “trickle” ventilators at the top of the window. These can be opened or closed to allow more or less trickle ventilation. If you close the window trickle vents the dMEV fans described in the previous section will still continue to draw fresh air into your home, however, particularly during the ‘running-in’ period we recommend that they are left fully or partially open to maximise the fresh air entering your home.

The following are general guidelines for your information.

To deal with condensation, take these two steps:

- Produce less moisture

Ordinary daily activities produce a lot of moisture very quickly.

Cooking:  To reduce the amount of moisture in the kitchen, cover pans and do not leave kettles boiling, open a window to allow excessive amounts of steam to be ventilated to the outside.

Washing clothes:  Put washing outdoors to dry if you can. Alternatively, please dry the washing in the bathroom (which is designed to accommodate drying clothes) with the door closed – the humidistat function of the extractor fan will help to remove the moist air to the outside (also if weather conditions permit – the bathroom window can be opened). If you have a tumble dryer, ventilate it to the outside (unless it is the self-condensing type).  D.I.Y. kits are available for this.

Drying clothes on radiator-mounted airers or on airers in rooms other than the bathroom may lead to excessive internal moisture.

- Ventilate to remove moisture

You can ventilate your home without making draughts.

Some ventilation is required to expel the moisture, which is produced all the time, mostly just by normal breathing of occupants. Keep a small window ajar or a trickle ventilator open when someone is in the room.

You need much more ventilation in the kitchen and bathroom during cooking, washing up, bathing and drying clothes. This means opening windows to assist in the ventilation of moisture to the outside and ensuring that the extractor fans are in full working order.

Close the kitchen and bathroom doors when these rooms are in use. This helps prevent the moisture reaching other rooms, especially bedrooms, which are often colder and more likely to get condensation.

COMMUNAL DIGITAL TELEVISION AND SATELLITE INSTALLATION

A television aerial socket has been provided within the Living Room and, depending on specification, other rooms (specific room may depend on house type, please refer to plans), to which a communal digital aerial and satellite signal will be fed. Your individual alterations may have requested additional points or a ‘returned’ signal to additional points in other rooms. Further information is provided in the Handover Pack.

The provision of a communal digital television and satellite signal means that the signal required for both “free” services, such as Freeview, and “upgrade” services, such as Sky (other providers are available) satellite television can be viewed, subject to the appropriate reception equipment and subscription being provided/installed by the home owner.

What does this mean?

Simply, there is no need (and the Deed of Conditions may prohibit) for multiple TV aerials/satellite dishes to be installed across the development, as this signal is supplied free of charge to the home, with maintenance of the common facilities falling under the responsibility of the factor. The TV signal can be regarded in a similar manner to any other utility, being delivered to the property just as water and electricity are.

The home owner then has the choice of what they want to do with the signal, a standard Freeview enabled TV will be simply “plug and go”, with many upgrade satellite services only requiring subscription/decoder box, but not additional installation/wiring.

Compatibility with Sky Q or other upgraded satellite services

At the time of writing, the communal satellite facility both within and outside your new home is ‘ready to go’ for the connection of services such as Sky +/HD, subject to subscription and receiving equipment being provided by you or your service provider.

In addition, the communal signal which is fed to your home is capable of delivering the feed required for upgraded services which are now reaching the market, such as Sky Q.

However, such services require an upgrade to the final connection hardware/infrastructure in order to provide the necessary distribution of signal. This can simply be ‘swapped out’ with standard equipment, which may need to be done by a TV services contractor before Sky or another provider makes final connections.

Taking Sky as an example, being the dominant provider, they may only provide a Sky Q compatible receiver box, which means the additional piece of hardware described below will definitely be required, even for a “non-Sky Q” subscription.

In most situations with a communal digital TV/satellite, you will find a fibre optic device, called a GTU (Gateway Terminal Unit), normally located adjacent to the electrical consumer unit within an individual property.

A device is now available which can be retro-fitted to this GTU in order to provide compatibility with Sky Q boxes, as shown below.

This product is a Triax 307368 TMDS 42C dSCR (digital Single Cable Router) multi-switch, which also retains the ability to deliver older Sky+/HD services.

What do I need to do?

If you are considering a new subscription with Sky, you will need to arrange installation of the multi-switch described above, which is not part of the communal installation. You have the following 3 options:

1.	 Should the standard terms within the legal missive of the purchase and construction timescales allow, you can request this multi-switch to be fitted as a client Extra, which would then be complete for the point of handover.

2.	 Where time does not allow for this be requested as a client Extra, and your property has been handed over to you, our Customer Care department/Home Owner’s Information Pack will provide detail of the contractor applicable to your development who would arrange an installation at your direct request.

3.	 Once the property has been handed over to you, you can arrange a contractor of your own choice to fit this/a similar product, details of which are provided in the Home Owner’s Information Pack.

Most reputable TV/satellite/audio-visual contractors will provide the installation service/s detailed above. A typical contractor used by Scotia is Campbell & Kennedy – www.campbellkennedy.co.uk.

Should any alternate/additional services and/or products be considered, we would also recommend engaging the services of a specialist contractor as per the example detailed above.

TELEPHONE INSTALLATION

The main telephone point is located typically in the Living Room area (specific room may depend on apartment type, please refer to plans), it is compatible with any BT approved phone.

It is your responsibility to arrange connection to your chosen telephone service provider and arrange final connection of secondary socket wiring to main point.

WINDOWS

Your property has white uPVC Sheerframe 7000 or 8000 system ‘Tilt + Turn’ inwards opening windows manufactured by CMS Window Systems Ltd, Castlecary.

Operating your windows;

Please refer to the manufacturer’s Operating and Maintenance Manual for information on opening your Tilt + Turn and fully reversible windows, also a copy of the relevant pages have been included below for your convenience.

Extract from the window manufacturer’s Operating and Maintenance Manual showing a typical Tilt + Turn standard window with the handle located at the side (note that the winkhaus restrictor catch is not fitted to your windows – see notes below);

Important Note for Tilt + Turn Windows - please note that the ‘Tilt’ opening function is intended for general day to day use. The ‘Turn’ opening function (where whole sash is opened into the room) is intended for cleaning and emergency escape purposes only. Care should be taken when using the ‘turn’ open function that the sash (or the trickle ventilator installed in the head of the sash) does not cause damage to the plasterboard window ingoes when fully open. The window should never be left unattended when open in the cleaning/escape ‘turn’ position.

Notes on window handles

The windows are fitted with lockable window handles to all ground floor windows and standard (non-locking) handles to all first floor windows.

A key is supplied to operate the lockable ground floor handles and care should be taken to prevent damage to the handle by trying to force it open when lock is engaged.

First floor window handles are non-locking (no removable key) to comply with the requirements of the Building Regulations in respect of emergency fire escape. However, the Building Regulations recognise that individual home owners may want to fit additional locking mechanisms to first floor windows after they have moved in to their new home (for example where there are small children in the room) and if you wish to install any of the large variety of ‘child restrictor catches’ or any other additional locking mechanism that are available to first floor windows then please note that they must be a ‘quick release’ type (without a key which might be lost) - a type which does not hinder escape through the window in the event of an emergency (a typical suitable example, a winkhaus restrictor for a tilt & turn window,  is shown in the above ‘tilt + turn’ extract from the window manufacturer’s operation and maintenance manual). The restrictor must also be suitable for the type of window and we also recommend that they are fitted by a skilled tradesman and that particular care is taken to ensure that they are fitted strictly in accordance with the restrictor catch manufacturer’s instructions. An incorrectly fitted additional locking mechanism can damage the window and lead to window guarantee problems. Advice can also be obtained from the window supplier (refer to the Schedule of Materials for contact information).

Maintenance and Cleaning Notes

Glass may be cleaned with either a proprietary household glass cleaner (following the manufacturer’s instructions) or a solution of soapy water. Glass can be easily scratched, therefore ensure heavy grime or dirt is removed carefully using soapy water.

uPVC frames should be cleaned every 3 months with a soap and water solution.

A non abrasive proprietary cleaner suitable for plastic may be used for more stubborn blemishes following the manufacturer’s instructions.

Avoid using solvent based or abrasive cleaners as these will damage the uPVC frames or glass.

Please refer to the CMS Operating and Maintenance Manual (contained within your Handover Pack) for more information including recommendations on lubricating and general maintenance.

Notes regarding glass coatings:

To comply with current building regulations all double glazed units installed in your home will have a low emissivity coating. Low emissivity (Low-e) glazing is a vital component of an energy efficient window or French door. It has a surface coating that allows short wavelength heat from the winter sun to enter your home through the glazing, while reflecting back into the room the long wavelength heating produced by your heating system. This reduces heating costs and minimises internal condensation. Please note that this Low-e coating has considerable advantages but you should be aware that there are some minor features, due to the coating of the glass, which you can see in some or all of the following ways;

•	As a tint in the glass

•	As a ‘haze’ when viewing through the glass at some angles and in some lighting conditions

•	By the appearance of condensation on the outside of the glass under certain weather conditions (which is positive proof that the glass is preventing heat loss from your home)

•	There may be minor blemishes visible arising from the coating process and the tint may also change between individual double glazed units if the units are made from different batches of glass. These are not detrimental to the functioning of the unit and are not a defect.

Please note that if you are replacing any of your double glazed units in the future you should ensure that your glazier uses low emissivity glass in your windows.

Note regarding glass specifications:

In addition to the note regarding Low-e coatings above, you should also be aware that certain windows may have either laminated or toughened safety glass installed. This ‘safety glazing’ is installed to comply with the Building Regulations. Any future replacement glazing units should be to the same specifications as originally fitted. Any competent glazier will be able to identify the glazing specification used and you should ensure that lower specifications are not used.

It should also be noted that windows with safety glazing have characteristics inherent to the nature of the product and its production process. Specifically you may find that the windows fitted with safety glass are heavier and when closing them from the ‘tilt’ position (for standard Tilt + turn windows) you may have to use both hands, placing one on the handle and the other on the window frame. This is not necessary where ‘easy reach’ tilt + turn windows are fitted.

In addition to weight considerations, while all double glazed units use ‘processed glass’ (and are subject to an amount of imperfections occurring during the manufacturing process), safety glass is more prone to these minor visual imperfections. An example of this is that when toughened safety glass is being manufactured it is heated to extremely high temperatures, creating micro-tears which on cooling add strength to the glass. This process can result in small imperfections or visual distortions forming on and in the glass- these are not usually noticeable under normal viewing conditions but may be evident under close inspection. They are, however, an acceptable characteristic of the finished product and are not recognised as a defect in the glass itself. Laminated glass, which is manufactured from several layers, has similar characteristics and it can also be subject to minor blemishes and imperfections which is an unavoidable consequence of the manufacturing process and is not a defect.

Important Safety Notes-Maintenance and Cleaning;

Care should be taken when opening and closing your windows for cleaning and maintenance- never lean out of the window or stand on a chair or other possibly unstable platform to reach the top of the window – use a suitable pole extension when cleaning the glass and window as shown in the diagram below. The window should never be left unattended when in the fully open ‘turn’ (for Tilt + turn windows) or fully reversed (for reversible windows) position.

Note regarding external sealant to windows and doors;

The windows (and external doors) have an external polysulphide sealant bead between the uPVC frame and the external render bead. This sealant bead is designed to provide a tough weather-tight seal to these joints. If any signs of deterioration of this bead are found it should be repaired or replaced with an equal specification exterior polysulphide sealant.

Important Note for First Floor Flats with Windows Over Flat Roofs;

The flat roofs are designed to accommodate weather related loadings such as snow and are not designed for any other additional loadings.

The flat roofs should not be used as a means of access, for example to clean the outside of your windows (the outside of the windows are designed to be cleaned from inside your property). They must not be used for any other purpose except as a means of escape in an emergency when, if necessary, you can escape from the flat through the window and across the flat roof. Please also note that the copings around the flat roof are made from lightweight GRP and can be easily damaged.

OPERATING INSTRUCTIONS FOR THE ELECTRICAL INSTALLATION

The Consumer Control unit for your property is located in the vestibule cupboard (ground floor apartments) or ground floor cupboard (first floor apartment). It contains labelled main isolator, RCDs and circuit breakers or “trip switches”.

The consumer control unit is a device that controls the electricity supply to your home, splitting the incoming electric supply into various electrical circuits around your home.

The consumer control unit contains Main Switch, RCBOs (Residential Current circuit Breaker with Overload protection), RCD (Residual Current Device) and MCBs (Miniature Circuit Breakers). The main switch is normally ‘ON’. In order to isolate all supplies, switch to ‘OFF’.

There are typically two RCDs in your consumer unit. Each RCD protects a section of the consumer unit. They are designed to ‘trip’ when there is an electrical leakage to earth thereby giving protection to personnel. An RCD would normally trip before an MCB.

These circuit breakers and RCDs are all designed to trip if there is a fault in a circuit, or if a faulty appliance is switched on. This helps to prevent serious accidents that may result in damage and injury. Under fault conditions these will be in the ‘tripped position’.

IF AN ELECTRICAL CIRCUIT FAILS

A circuit may trip OFF. If this happens, you should follow the procedure set out below.

1.    Check with the aid of a torch whether the RCD (mid position) or MCB (fully down) is in the OFF position.

2.      Switch RCD (press down then push to the fully up position) or MCB to ON position.

3.      If the RCD does not re-set, switch off all the MCBs, re-set the RCD then switch on each MCB individually until the faulty circuit is identified.

4.      To identify the cause of the fault switch off all appliances in that circuit, re-set the RCD and MCB, then switch back on each appliance until the defective appliance is found.

Over-filling kettles, irons etc. can cause this type of fault.

N.B.

It is important to ensure that the bulbs used in light fittings do not exceed the rating for that fitting.

NOTE:  Electricity is dangerous and can kill. If you are unsure of any aspect of your electrical installation, please consult a qualified electrical contractor.

SMOKE, HEAT & CARBON MONOXIDE DETECTORS

Your property is fitted with ceiling mounted smoke detectors (typically in living rooms and halls), a CO (Carbon Monoxide) detector (typically in the same room as the boiler) and also a Heat Detector (typically located in the kitchen area). The locations and quantities of these detectors depends on your apartment type.

These alarms are mains operated with battery back up and connected to bedroom lighting circuits. The smoke detector is extremely sensitive to smoke and dust particles of any kind and can be activated by the likes of burning toast. The heat detector in the kitchen area is less likely to cause ‘false alarm’ problems as it is not responsive to any type of smoke or fumes, only heat such as generated by a chip pan type fire (but not from smoke caused by burning toast or similar). The CO detector monitors Carbon Monoxide levels and its alarm will activate if safe levels are exceeded.

You must read and fully familiarise yourself with the instructions for the smoke, heat and CO detectors. Copies of the instructions are contained in your hand over pack and are also available for downloading from the manufacturer’s website- the instructions contain vital information on the operation and maintenance of your detectors.

If the smoke detector activates you should check the property and, if no reason for its activation is found, it could be a nuisance alarm caused by cooking smoke reaching it or something similar. If this occurs, open a window to clear the smoke or dust and the alarm will cease and test/ maintain the detector as described in its instructions.

If the heat detector activates you should follow the advice contained in the heat detector instructions.

If the CO alarm activates please carry out the instructions contained in your carbon monoxide alarm instructions. Depending on the type of CO alarm fitted these instructions typically include the following -ventilate the area (open windows and doors), turn off all fuel appliances where possible, evacuate the property, get medical help for anyone suffering from the effects of CO poisoning, ring your gas supplier or other supplier on their emergency number, do not re-enter the property until the alarm has stopped (if the alarm has been silenced by pressing the Test/Hush button, wait at least 5 minutes to allow the alarm to check that the CO has cleared). Do not use the fuel burning appliance(s) again until they have been checked by an expert. In the case of gas appliances this must be a Registered Gas Installer.

To reset or to test the smoke, heat and CO detectors follow the manufacturer’s instructions as enclosed in your Handover Pack.

The back up batteries should be changed as recommended by the manufacturer and an intermittent beep normally indicates that the battery needs to be replaced. The CO detector sensor module typically must be replaced after 5 years of operation (refer to its instructions).

EXTERNAL DOORS

All ironmongery should be cleaned on a regular basis using warm soapy water. Do not use abrasive or corrosive material to clean the ironmongery as this will damage the finish to handles, letter plate, eye viewer, chain and rain deflectors.

Glazing can be cleaned with warm soapy water, avoid using anything which may scratch the glass.

The multi point locking system should be lubricated with WD40, or a similar product, on a regular basis to ensure the smooth operating capabilities of the cylinder, handle and the locking mechanism.

The weather sill at the base of the door should be kept clear of debris to allow the drainage holes to function.

The rubber gaskets in the sill and door frame should also be checked periodically for damage and replaced as required.

INTERNAL DOORS

Handles should be cleaned with a soft non abrasive cloth and for stubborn stains mild soapy water may be used.  Care should be taken to avoid scratching surface of handles.

The mechanism of the handle should be lubricated once a year with a light oil.

Hinges and latches/locks should be lubricated on a regular basis with WD40 or similar product.

GARAGE DOORS (where applicable)

If you have a garage with your property then typically a Hormann sectional garage door has been fitted to your garage (where applicable).

Please refer to the Fitting, Operating and Maintenance instructions for this door contained in your hand over pack (copies of which can also be downloaded from the Hormann website) for full operating and maintenance instructions and note also the following important points;

- The garage door should be maintained in accordance with the manufacturer’s instructions.

- Always keep the opening area of the door clear and when in operation make sure that neither persons, children in particular, nor objects are located within the door area of travel.

- Operation of the during heavy winds may be dangerous

- Have a specialist inspect and maintain the door at least once a year (or every 6 months if there are more than 50 door cycles per day)

Warning Notices from manufacturer’s operating instructions;

WALL TILING

Wall tiles and in particular the grout between tiles should be regularly cleaned using a proprietary tile/grout cleaner in accordance with the manufacturer’s instructions.

Grout should be inspected and any areas which become loose should be replaced.

The sealant between the wall tiling and any worktops should be inspected and replaced as necessary.

SHOWER WALL PANELLING (where fitted)

Laminate wall panelling (where fitted) should be cleaned by using hot water and a mild detergent applied with soft cloths or soft nylon brush.  Non scratch cleaners may also be used. On no account should scouring pads, acid based toilet cleaners or limescale cleaners be used.

Wall panelling and shower enclosure/tray should be dried off after use.

Abrasive or aggressive cleaning products should not be used as they will damage the laminate surface of the panel.

The sealant around the base of the wall panel should be inspected and replaced as necessary to prevent water ingress between the shower tray and the panelling.

INTERNAL DRAINAGE

Soil and vent stacks run vertically through your apartment and they also serve your neighbouring apartment (either above or below as applicable to the location of your flat). The soil stacks are hidden within plasterboard faced ducts and bulkheads and have access panels located at points where access may be required in the future if maintenance work is being carried out.

Picture of a typical plastic access panel;

These access panels are recessed into the plasterboard and have a hinged fully removable door for easy and convenient access. They can be painted over, if required, during any future re-decoration work that you may carry out. Please note that these access panels should not be removed or sealed up – they are there to allow fast access to rodding or drainage access points – access will be required if, for example, a blockage occurs in the soil pipe or during maintenance works.

FLOOR FINISHES

Please note that any wooden flooring or other feature flooring such as tiles or adhered ‘Karndean’ (or similar flooring) laid by you when you move into your home will not be lifted and re-laid as a result of any maintenance work which may require access under the floor. If maintenance work is required to any part of a floor or to any under-floor services we will require you to arrange the lifting and reinstatement of any feature flooring to allow us access.

Notes regarding concrete floors (normally ground floors) – concrete floors in your home have been finished to the standard tolerances obtainable by the material. Before laying floor coverings such as vinyl or wooden overlay flooring to concrete floors you should be aware that some important steps should be taken by you;

- You should have your floor covering installer check the moisture content of the concrete floor. This is particularly important if you are laying the floor covering immediately after moving into your home, as the concrete floor may still be drying out and moisture can affect some flooring materials. If necessary you should allow the concrete floor to dry out sufficiently before laying any flooring which may be affected by moisture or the concrete floor should be treated in accordance with the flooring supplier’s recommendations before installing any vinyl or wooden or other feature type flooring.

- Concrete floors will normally need a self levelling screed applied prior to laying any vinyl or overlay type floor covering, again in accordance with the floor covering installers recommendations.

Notes regarding Chipboard Flooring - Chipboard flooring (normally to the first and second floors) should be prepared in accordance with manufacturer’s recommendations prior to fitting of vinyls, wooden overlays or ceramic floor tiles. Also note that chipboard flooring may have service ducts installed (sections of flooring which can be removed to allow access to pipes and other services). We have installed these ducts so that they are level with the adjacent floor – however it should be noted that, because timber floors naturally shrink as they dry, this drying out may result in minor differences in level between the duct cover and the surrounding floor. This may become evident with some types of thin floor coverings and your choice of floor coverings should take this possibility into account.

Note regarding installation of carpets to staircases – carpet smooth edge/gripper strips must be glued or screwed to timber staircases – not nailed. Scotia will accept no responsibility for risers damaged due to nailing of carpet grippers.

ROOF SPACE (Where applicable)

The attic space (where applicable to your property) has not been designed to allow for storage. Do not use the attic space for storage. Flooring the attic space and using it for storage may cause deflection in the roof structure resulting in problems such as cracks appearing in your ceiling.

The attic has mineral wool insulation between and over ceiling joists.  This insulation can cause skin irritation. If handling the insulation it is recommended that appropriate protective clothing and equipment be worn.

Care should also be taken if entering the attic – the ceiling plasterboard between the joists will not support your weight and there may be service pipes, extract fan ducting and cables hidden by the insulation that you can damage by inadvertently stepping on them.

An attic access hatch (also called a loft access door) has been provided through the ceiling of top floor flats and this is intended only to be used by tradesmen for attic space inspection or maintenance work. Note that if opening the loft access door it should be supported so that it does not swing down suddenly.

Operating instructions are normally to be found fixed to the inside of the ceiling loft access door- the following photograph shows a typical set of instructions;

Top floor flats may also have a second, smaller inspection hatch installed in the ceiling (these are to allow access for inspecting the boiler flue in the attic and are for your heating engineers use when carrying out the annual boiler service. Note this is only applicable to some flat types.

It is advised that you do not enter the attic space.

The attic access hatches should only be used by competent tradesmen for access to the attic space for any essential maintenance works.

CONSTRUCTION OF WALLS, PARTITIONS, FLOORS & CEILINGS

The following notes provide, for your information, outline details of the construction for each part of your home. This information is generalised and particular areas of your home may differ – always seek appropriate advice and carry out detailed investigation works before making any alteration to your home in the future.

External walls:	The external walls comprise 100mm thick block-work (with render finish), 50mm wide cavity and EcoWalltf timber framed kit inner leaf. The timber framed inner leaf comprises; a reflective breather membrane on the cavity side on a 9mm sheathing board, on 140mm thick load bearing timber framing, with 140mm thick mineral wool insulation between the studs, 30mm PIR insulation fitted to the inside of the studs, and a reflective vapour control layer fitted on the inside face of the PIR insulation. On the inside of the timber kit there is a 38mm timber framed service void and 2 layers of 12.5mm thick plasterboard lining. Safety Note – all external walls are load bearing and designed to achieve apartment fire and acoustic building regulation requirements and must not be altered without getting professional advice.

Indicative diagram of a Typical EcoWall timber framed external wall for your information (note that apartments have two layers of plasterboard internally);

Party walls:	(Walls between your property and adjoining neighbouring property - where applicable- note that the wall specification varies from the following if your property is adjoining a 3 storey block of flats). These walls comprise two 90mm thick timber panels with 9mm sheathing boarding to the cavity side and a 50mm cavity between the panels. Acoustic insulation is fitted between the timber studs. On the room side a layer of 19mm thick plasterboard plank is fitted and then this is over laid with 12.5mm plasterboard. Where there are sockets or other items of electrical services installed on the party walls a service void is also formed using 38mm thick timber framing overlaid with a layer of 12.5mm plasterboard lining. Safety Note – all party walls are load bearing and are constructed to a fire resistant/ acoustic specification – they must not be altered in any way. If any damage occurs to plasterboard and plank linings to the house side of the party walls the damage must be repaired immediately.

All partitions:	Timber framed partitions with 15mm thick plasterboard each side. Mineral wool acoustic insulation is fitted in the partitions to bedroom, bathroom and the like areas. SAFETY NOTE – some internal partitions are loadbearing and have two layers of plasterboard fitted to each side. Do not remove or alter load bearing partitions, or make substantial alterations to them, without getting professional advice.

Ground floors:	Concrete floor slab on rigid insulation. The ground floor also incorporates a carbon dioxide membrane located under the floor – please refer to the Section ‘Carbon Dioxide Membrane’ for more information.

First floors (apartment separating floors);

Separating floors comprise engineered I joists with a 15mm decking board on top of the joist with dynamic floor battens, acoustic insulation between the battens, 19mm thick plank and finally an over-lay of 22mm thick chipboard flooring. To the underside of the I joists are 2 layers of 15mm thick plasterboard fixed to a resilient bar system which is fixed to the joists.  A layer of acoustic insulation is installed between the joists. Safety note – all apartment separating floors are load bearing and designed in accordance with the building regulations in respect of fire and acoustic requirements – the separating floors must not be altered.

Top floor	 Plasterboard fixed to the underside of the roof trusses.

ceilings:

Future alterations – should you consider making any alterations to your home in the future such as altering the partition layout or forming a new opening through a wall you should check relevant Local Authority permissions and/or use the services of a qualified architect before starting.

The external wall service voids, internal partitions, 1st floors and attic space all have services such as pipes and cables installed in them – refer to the safety precautions below if installing any fixings into these parts of your home.

Appropriate proprietary fixings should always be used to suit the wall construction (see below).

FIXING TO WALLS, CEILINGS OR FLOORS – IMPORTANT NOTICE

Wall fixings (for pictures, mirrors etc.) must be of the appropriate type for the type of walls described above. Be very careful if nailing or drilling into walls, ceilings or floors to avoid contact with any pipes or electric cables which may lie hidden behind the surface. We recommend that you use a services detector (cable detector) before drilling or nailing – it can reduce the risk of serious injury. If using power tools to install a fixing, you should always use a R.C.D. (residual current device). You should also always check for pipes and cables before drilling or nailing into floors or ceilings.

Note for any apartments with under-floor heating (if applicable) - you should never drill or nail into any floor which has under-floor heating fitted.

In addition to the above please note that fixings should never be made to the following wall areas:-

a)	Directly above or below any electrical socket outlet, switch or appliance.

b)	Directly horizontal to any electrical socket outlet, switch or appliance.

This is because electrical cables run in these areas.

external fixings

Any external fixings should only be made with consideration to the Deed of Conditions.

high performance keim masonry paint

Some exterior features of your home such as pre-cast concrete window sills, base course block-work and in some instances the feature window and door surround bands may have been painted with a Keim Mineral Paints Ltd paint system. Keim mineral silicate paint systems were invented over 125 years ago to provide long term protective and decorative finishes for renders and masonry subject to harsh climatic conditions.

This specialist paint system has a considerably longer life cycle than other masonry paints. Manufacturer’s studies have shown that redecoration over a 30 year period was required once for the Keim paint system (after 16 years), whereas between 3 and 9 redecorations were required for other masonry paints.

K REND exterior features

Some exterior features of your home such as the external bands around windows and external doors have been formed with pre-coloured K Rend Silicone FT render.

K Rend Silicone FT is a water repellent cement based and polymer modified self coloured render which provides a high quality durable weatherproof coating with a natural finish.

MOCK CHIMNEYS

Lightweight mock chimney’s (where fitted) are manufactured by Capvond Plastics Ltd of Glasgow. The mock chimney’s have been independently tested in respect of wind load resistance, hygrothermal test (extremes of heat/rain and freeze/thaw cycles) and water ingress and passed the strictest testing requirements.

The chimney stack has a render finish applied to the outside. The chimney cope and pots are GRP.

Note that the mock chimney is not designed to support additional loadings such as satellite dishes or TV aerials or other such apparatus. Do not install any fixings to the mock chimney.

EFFLORESCENCE

The appearance of a white deposit on external walls is caused by ‘efflorescence’.  This is a consequence of drying out and can often occur after a new house is constructed and is drying out. It can also occur when a wall dries out after period of heavy rain or in the spring as a result of drying out after a wet winter. As well as external wall materials such as block-work and mortar joints, it can also occur on products such as precast window sills, driveway paviors and paving slabs and also internally on concrete floors and areas of similar construction.

The efflorescence is caused by natural salts being drawn out of the wall materials while drying out and is quite normal. It is neither harmful nor detrimental to the performance of the material and, whilst it may look unsightly, the majority usually disappears over time. The advice given by most brick, block, cement and precast concrete manufacturers is that it is best dealt with by the combined effect of time and weather. If efflorescence occurs externally on your home it is our policy to follow this advice and allow it to disappear naturally. It will usually disappear within a few weeks, washed away by normal rainfall. This process may take some time to draw out and remove all of the natural salts causing the efflorescence however it should be apparent that each time the efflorescence appears it will be in decreasing amounts.

If efflorescence occurs on internal concrete floors or other such areas then it can be removed by brushing with a non metallic brush and then removing the deposits with a vacuum cleaner. Internally occurring efflorescence should disappear quicker after brushing and vacuuming than external efflorescence as the home is dried out by the heating.

Should persistent efflorescence occur internally which does not disappear after removing it by the methods described above, then please contact Scotia for further advice.

EXTERNAL AREAS

Manholes give access to the drains, usually where branches join together, do not obstruct or cover them with soil. You may need to provide access to them quickly if there is a blockage. Please note that there are live underground services cables in the ground around your house. Great care MUST be taken if digging or carrying out excavation work in the vicinity of live underground cable routes.

Underground cables may be found just below the surface, although they are normally laid between 0.45m to 1.0m deep from the surface. Reduced depth may result from ground disturbance after laying or because the cable had to be laid over an underground obstruction. Even shallow excavations (e.g. for post holing and fencing work or for garden features such as ponds) may be a source of danger.

If you do uncover a cable during excavation work - ALWAYS assume it is live. If in any doubt contact a qualified person to seek advice before carrying out excavation work.

Garden and Exterior Maintenance - Caring for your garden.

Caring for the grass (including trees and shrubs where they have been provided) in your garden is essential. This will ensure that the planting is successfully established and your garden thrives.

The rear gardens of most new homes are finished in roto-vated topsoil, allowing you the opportunity of designing and landscaping to your own requirements. It is important that the landscaping is carried out as soon as possible after the date of the handover, as it is only by working the soil that it will remain aerated and weeds will be prevented from becoming established. This will also help to establish the finished level of the soil and ensure it drains more effectively- so reducing the potential of any flooding during wet weather conditions. However, if there has been rain please take this into consideration when carrying out landscaping work to your rear garden – let the topsoil dry out sufficiently before working on it - compacting wet topsoil will damage it making it unsuitable for good grass or plant growth.

The front gardens are normally turfed or may have bark chippings and also may have shrub or trees planted in them. There are some important care and maintenance requirements, particularly in the first year after handover (note – not all of these requirements may apply to every plot):

Watering – In the absence of regular heavy rainfall you should water your turf at least twice a week – daily if the weather is hot and dry – after moving in. A newly turfed garden looks deceptively mature but the new grass has only a very small reserve of moisture in the soil attached to the turf. Until the grass roots grow into the underlying soil the turf is prone to drying out and shrinking. This can leave unsightly gaps. Light rain is often not enough to sufficiently water the turf and underlying soil. The best way to irrigate your garden is by using an oscillating sprinkler. If your property has metered water you may be put off by the cost of using a sprinkler but it is likely to be much more costly to rectify problems caused by drying out.

Trees and shrubs also need copious watering after planting. As with turf, the roots have not yet grown into the surrounding soil and can only pick up moisture from a very limited area. Planning permission for planting schemes usually requires that planting shown on the approved plans is maintained or replaced for a specified period of time. This obligation is passed on to you once you take up ownership of the property. Scotia does not replace turf, trees or shrubs that have failed due to a lack of watering.

Damage – Walking on turf before it has properly settled in can cause considerable damage. Dents and hollows made on new turf will not disappear and are often difficult to repair. It usually takes about a month to become firm enough to walk on, but this can vary according to weather and soil conditions. If you are installing or removing a sprinkler use wide boards to spread your weight and minimise damage.

Mowing – it is recommended that you do not mow the turf for at least the first week after moving in. Let it grow to establish itself and make sure before mowing for the first time that its roots have grown down into the soil below.

For the first cut leave the grass higher than normal and then gradually reduce the height in subsequent mowings until you reach the height you want.

Bark Chippings – decorative bark chippings will need ‘topping up’ on a regular basis to keep them looking their best.

Fencing – in some cases it is possible that a rear or side boundary is shared with other plots on the development and fencing has previously been erected at the time of completion of those plots. Under normal circumstances, this will not be replaced and may be subject to natural weathering, resulting in colour differences which are not considered to be a defect. Where joins/extensions require to be made to existing fencing, any remedial works will be undertaken, but this will not include the replacement of whole panels/sections, unless they are defective.

Rotary Clothes Dryer – If a rotary clothes dryer has been provided, please note that children should not play with this product – it is recommended that when not in use it is folded and stored safely out of the reach of children.

Damp Proof Courses – there are damp proof courses built into your external walls to prevent damp from the ground soaking up the outside walls. These are normally approximately 150mm from the ground level around your house. It is important that these damp proof courses are kept clear – if you are carrying out any landscaping or ground-works alongside your external walls then please ensure that you do not cover these damp proof courses or otherwise bridge them, allowing damp to rise up past the damp proof course.

CARBON dioxide MEMBRANE

There is a carbon dioxide gas membrane under the concrete ground floor and an external gravel filled trench around the perimeter of your home which allows any carbon dioxide to disperse to the outside air.

Should you intend to carry out any work involving alterations to your ground floor which would involve puncturing the carbon dioxide membrane then it should be protected/ reinstated in accordance with the manufacturer’s recommendations.

The gravel filled trench around your home should not be altered – do not remove it and replace it with an impermeable surface such as paving slabs, or cover it with topsoil. If the gravel surface of the trench becomes blocked with silt or soil then the gravel should be cleaned or the top layer replaced with clean gravel to ensure that any carbon dioxide can disperse to the outside air.

ROOF rainwater sYSTEM

The rainwater from your roof is piped to a communal SUDS (Sustainable Urban Drainage System) detention basin facility in the North East corner of the development. No maintenance is required by yourselves to the pipework from the disconnecting manhole on the boundary of your plot to the SUDS facility and the SUDS facility itself is maintained by Scottish Water/ Aberdeen City Council.

SURFACE AND RAIN WATER DRAINAGE CONSIDERATIONS

We have designed the rainwater and any driveway drainage to comply with Local Authority Regulations (such as planning, building control and roads construction consent conditions) and SEPA (Scottish Environment Protection Agency) requirements including SUDS (Sustainable Urban Drainage Systems) requirements. In simplified terms these regulations require us to ensure that the drainage systems designed and installed around your home collects any rainwater which falls onto your house and surrounding plot and drains it away in a responsible manner.

It is common for the regulations to require that the rainwater falling on your garden ground is attenuated within your plot boundary and encouraged to soak away into the surrounding ground rather than running off your plot into the local authority drainage system. This practice minimises the risk of the local authority drainage systems becoming over loaded in periods of high rainfall therefore reducing the possibilities of flooding.

There are a number of very important points that you should be aware of in relation to the drainage around your home;

- Alterations to your driveway or parking spaces (or other areas within your plot curtilage). It is very important that rainwater does not run off your plot onto the adjacent roads and footpaths (this is particularly important where the road is adopted by the local authority). We have designed the access driveway to ensure that any rainwater falling onto it either runs off into your plot where it soaks away into the ground or is collected into a gully or a permeable surface is used on the drive (such as gravel or permeable blocks). If you subsequently make any alterations to your drive you must bear this in mind and make sure you have obtained the necessary permissions from the relevant local authority. For example if you have a gravel drive which slopes down to the road outside your home and you decide to have it tarred then you will also have to install suitable drainage to deal with any rainwater which falls onto the drive. Failure to make such drainage provision will be likely to lead to the local authority demanding that the original surface be reinstated. Also if you extend your drive over garden ground you must ensure that existing drainage provisions are adequate. Finally, it should also be noted that if you are changing the surface of your driveway the local council may require that the first two meters of your drive must be a ‘hard surface’ – not stone chippings or gravel or similar loose surface- again it is important that you obtain the necessary permissions prior to making any alteration to your drive.

- Alterations to your garden ground. Removal of garden areas and installation of, for example, large impermeable patio areas or a large area of other hard standing will reduce the area of ground available to soak up rainwater and could lead to flooding problems if adequate drainage is not installed at the same time. If you decide to ‘slab over’ your garden ground you must also ensure that you make adequate provision for dealing with any rainwater to avoid increasing the risks of flooding your own and your neighbours properties.

- Maintenance of the drainage system. It is essential that the drainage provisions installed around your property are maintained to keep them in good working order. This not only ensures that any rainwater which falls onto your house and plot dealt with efficiently, but will ensure that risk of flooding to your home and surrounding properties is minimised. Depending on the particular drainage systems installed around your home the following general maintenance notes should be adhered to (where applicable);

- Permeable block drives and parking areas (where installed) – Permeable blocks have small gaps between each block to allow water to drain between the blocks and into the layers of material below and ultimately into the surrounding sub-soils. For this to remain effective you should ensure that you do not allow the gaps between the blocks to become clogged with fine soils or other debris. Generally most permeable block manufacturers recommend sweeping the blocks twice a year to remove any loose particles from the surface is adequate and if the gaps do become badly clogged they can be cleaned out with a suitable tool. However please note that generally these porous blocks are designed to deal with a much higher rainfall intensity than normally encountered in the UK therefore their continued operation can accommodate a reasonable amount of debris in the gaps of the blocks. Power washing of the blocks is not recommended because it can lead to the fine bed that they are laid on becoming eroded and rutting, depressions and cracking of the block surface can arise as a result.

- Gravel drives (where applicable)– these are generally maintenance free and only need raked level on occasion to remove any rutting caused by cars or footpath traffic and the gravel may need ‘topped’ up from time to time to keep it looking at its best.

- Garden ground – please refer to the maintenance information contained in the previous ‘External Areas’ section.

- Avoiding blocked drains. The foul drainage system from your home is designed to take used water from sinks, showers, baths and toilet waste. The drainage is not designed to take inappropriate items such as wipes (baby, personal cleaning and the like), sanitary items, cotton wool, cotton buds, disposable nappies, cooking fat or oil or grease and similar substances.

Blocked drains can lead to flooding of your property and your neighbouring properties. A majority of blocked drains are caused by inappropriate items being put down the toilet or fat, oil or grease being put down the sink. Please refer to the Scottish Water material on the following pages for more information.

- Water Butts. If you decide to install a water butt to one or more of your rainwater downpipes please ensure that you also fit an over-flow back into the rainwater downpipe (to avoid the water butt over-flowing and causing flooding) and that any water butts are located in accordance with any relevant Deed of Conditions. Kits for water butt overflows are available in any good garden centre.

METERS

The Electric meter is located in the vestibule cupboard or understair cupboard (depending on the apartment type).

The Gas meter is located in an external semi-concealed ground meter box.

Utility suppliers

The existing suppliers for gas and electricity to your new home will be recorded and your understanding of this confirmed on the ‘New Home Introduction’ form. Suppliers are required to be in place prior to completion for the purposes of installing infrastructure, making connections, and testing various features of the property. At the time of writing, the supplier for both gas and electricity (where applicable) will be British Gas at the point of handover.

Once you have taken possession of your new home and ownership has been legally transferred, it of course becomes your own right to decide which particular supplier you wish to use. It is at this point that meter readings are recorded on handover documentation, thereby denoting the change in responsibility for payment of ongoing bills. This same documentation, specifically the Handover Certificate, also includes MPAN (Meter Point Administration Number) and MPRN (Meter Point Reference Number) details, which are determined well in advance of construction completion and provide supply point identification for your new home.

It is strongly recommended that you keep this Handover Certificate in a safe place.

Utility suppliers are then provided, by us, with these readings and confirmation of change of responsibility in order that they may start to invoice you directly as a private customer and homeowner.

From this point, it is possible for you to review, and indeed change, the tariff and/or supplier for gas and/or electricity. Although the administration of the changed contact details may take a little time for some suppliers to update, all that is required to process a change is simply the MPAN and/or MPRN details as a starting point.

Whilst there is an inevitable time period during which specific contact details applicable to an address are updated to current suppliers and this is subject to administrative process, the use of correct MPAN/MPRN details as noted above should provide sufficient reference for review and/or change of supplier after the point of handover.

N.B.	Some developments may already be fitted with ‘smart’ meters, which allow for both remote taking of meter readings and a separate display of usage and costs. These meters are progressively being installed in both new and existing properties, and will become prevalent in the near future. Should your new home be fitted with a ‘smart’ meter and you choose to change supplier, please be aware that you may lose some ‘smart’ functionality if the chosen supplier is not yet compliant with this technology.

LOCAL AUTHORITY REFUSE AND RECYCLING COLLECTION

The development has been planned to incorporate the required storage stances for wheeled bins and routes for collection vehicles. Aberdeen City Council has responsibility for refuse collection at Charleston, Cove and, at the time of writing, requires the developer to organise and provide appropriate bins to each plot on completion. It remains your responsibility for making these bins available for collection on the designated days in accordance with good practice and the Council’s policies.

Should you have any queries or need advice regarding Waste and Recycling, perhaps for additional bins, advice on special collections or waste collection calendars in your area, please contact the Aberdeen City Council Waste Team on 0845 6080919 or wasteaware@aberdeencity.gov.uk, http://www.aberdeencity.gov.uk

council tax

The local authority will be aware of the new homes which are within your development, with a responsibility for payment of council tax falling upon the new owner (yourself). The authority will have made a banding valuation for your own property type and will issue payment instructions and schedules accordingly.

Should you not receive confirmation of this from Aberdeen City Council or have any questions, please contact their Revenues and Benefits team at:

Corporate Governance
Business Hub 16
Third Floor West, Marischal College
Broad Street
Aberdeen

AB10 1AB
Email: counciltax@aberdeencity.gov.uk	Website: http://www.aberdeencity.gov.uk

Fax: 01224 346700

SCHEDULE OF TEST CERTIFICATES

ALPHA INTEC GAS BOILER

Installation and servicing instructions (including service record) are contained in your hand over pack.

SCHEDULE OF MATERIALS

| Item | Description | Supplied by | Tel No. |
| --- | --- | --- | --- |
| Structural timber frame | Structural timber frame kit including external wall panels, load bearing internal partitions, separating floor structures and roof structure. | Supplied and erected by Deeside Timberframe, Stonehaven | 01569 767123 |
| Windows | White uPVC inwards opening Sheerframe 7000 or 8000 tilt and turn windows | CMS Window Systems, Castlecary | 01324 841398 |
| Internal Doors Leafs | Jeld-Wen Arlington 6 panel smooth heavy weight | International Doors & Windows | 01224 682229 |
| External Doorsets | Smith & Frater Ltd GRP Doorsets | International Doors & Windows | 01224 682229 |
| Garage doors (where applicable) | Hormann Sectional garage door | Supplied and installed by Garage Door & Gate Co (North East) Ltd, Kintore | 01467 633624 |
| Skirting boards & Facings | MDF ogee 7 skirting boards and Ogee facings | Fleming Buildbase | 01224 258200 |
| Ironmongery for internal doors | Heritage Brass Windsor V710 lever handles to room doors and V730 to bathrooms (with Legge bathroom lock), in polished chrome finish | George Boyd Ironmongery, Aberdeen | 01224 685541 |
| Kitchen Units & Worktops | Laings Directline range with Duropal worktops. | James Laing & Son Ltd, Inverurie, Aberdeenshire | 01467 620311 |
| Kitchen Appliances | Various (depending on plot) | James Laing & Son Ltd, Inverurie, Aberdeenshire | 01467 620311 |
| Kitchen sink | Leisure Euroline stainless steel inset sink 1 ½ bowl | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Sanitary-ware(standard specifications) | Ideal Standard Alto 55cm basin and semi pedestal to bathroom and Alto 45cm wash hand basin to cloakrooms. Ideal Standard Tempo Cube 1700 x 700 bath. Ideal standard alto WC pan close coupled with Alto cistern and Alto seat and cover. | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Taps | Bristan Ruby mono bloc sink mixer (kitchen), Bristan Prism basin mixers and Bristan Prism mono bath filler. | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Shower Valve | Bristan Prism Thermostatic Shower Valve CP | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Shower Head & Rail | Bristan Quadrant Shower Kit CP Ref. kit2c- CP | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Bath Screen | Novellini Aurora 3 | William Wilson Plumbers Merchants, Aberdeen | 01224 335328 |
| Central Heating + Hot Water System | Alpha Intec 34C and associated components | Northern Heating | 01224 663322 |
| Radiators | Myson Premier HE | Northern Heating | 01224 663322 |
| Radiator Valves | Danfoss RASC2 (10mm or 15mm) | Northern Heating | 01224 663322 |
| Switches & Sockets | Click Mode Range | Holland House | 01224 638129 |
| Extract Fans | Greenwood Airvac Unity CV2GIP Dmev mechanical extract fans | Holland House | 01224 638129 |
| Paving slabs | Grey Riven by Concrete Products (Kirkcaldy) Or Stonemarket Ryton (riven finish) grey utility paving slabs | GPH Builders Merchants Ltd., Inverurie | 01467 621926 |
| Pavers (where not permeable pavers) | Marshalls Driveline 50 | Keyline Builders Merchants, Dundee | 01382 448600 |
| Roof Tiles- ‘grey’ roofs | Marley Edgemere interlocking tiles. Colour smooth grey with Marley Modern ventilated dry ridges and European Plastics Ltd Type M continuous dry verge (where applicable) | Roof Tiles and associated ridge and verge materials supplied and installed by Forster Roofing Specialists Ltd, Brechin Angus | 01356 628560 |
| Roof Tiles – ‘red’ roofs | Sandtoft 20/20 interlocking tiles. Colour natural red with Sandtoft half round ventilated dry ridges and Sandtoft segmental verges (where applicable) | Roof tiles and associated ridge and verge materials supplied and installed by Forster Roofing Specialists Ltd, Brechin Angus | 01356 628560 |
| Basecourse | Fair Faced Grey Blocks (chamfered to public elevations) | Fyfestone Masterbock | 01467 651000 |
| CO2 Barrier (under ground floor) | Visqueen CO2 (carbon dioxide) membrane (and associated components including damp proof course and top hat units) | Keyline Builders Merchants, Dundee | 01382 448600 |
| Cast Stone Dressings (window sills, feature lintels and door canopies – where applicable) | Plain Grey Smooth Precast concrete | Montrose Precast | 01674 677037 |
| External walls Render | Dry Dash Roughcast with colours depending on the specific plot as follows; Buttercream – buttercream drydash roughcast backing coat with a 4 to 6mm quartz chip. White – Snowcrete white drydash roughcast backing coat with Essno white chips. Grey – Cement Grey drydash roughcast backing coat with silver grey chips. | Materials supplied and installed by Lawson Roughcasting, Stonehaven | 01567 973467 |
| K Rend (feature render bands around windows and doors where applicable) | Smooth K Rend Silicone FT render, colours vary depending on plot as follows; Powder Blue Pewter Grey Champagne White | Materials supplied and installed by Lawson Roughcasting, Stonehaven | 01567 973467 |
| K Rend (product manufacturer) | Contact details for product manufacturer | Kilwaughter Chemical Company (K Rend), N Ireland | 028 2826 0766 |
| Mock Chimneys | Lightweight GRP chimneys | Capvond Plastics Ltd, Glasgow | 0141 876 9000 |
| Keim Masonry Paint (to window sills, base-courses and the like- where applicable) | Keim Soldalit masonry paint. Colours vary as follows; Light Blue = Keim 9490 (used with powder blue K Rend surrounds) Off white = Keim 9295 (used with Champagne K rend surrounds) White = Keim 9872 (used with white K rend). Note – where pewter grey K-rend is used then the sills, base-courses and the like are not painted. | Keim Mineral Paints Ltd, Telford | 01952 231250 |
| Gutters & Downpipe | Marley Deep-flow colour black | Drain Centre | 01224 626497 |
| Paint to walls and ceilings | Dulux Trade Super Matt Emulsion | Dulux Decorator Centre | 01224 573044 |
| Paint to skirting boards etc. | Dulux Eggshell | Dulux Decorator Centre | 01224 573044 |

Note - Not all items or colours are applicable to all properties

Scotia Homes Limited

Balmacassie, Ellon, AB41 8QR ● Tel: 01358 722441 ● Fax: 01358 723499

Email: info@scotia-homes.co.uk ● www.scotia-homes.co.uk