# Customer Specification

Brechin West, Brechin

# Walls

External walls will be of timber frame construction with an external leaf of either rendered blockwork or feature panels of Gleneagles Granite grey coloured facing brick to match the basecourse.

Internal walls will be of plasterboard finish mounted on timber studs.

# Roof, fasciae & guttering

Roofs will be finished in Marley Edgemere smooth slate grey interlocking concrete roof tiles. Where applicable, fasciae will be in white finish uPVC, with guttering/downpipes in black finish uPVC.

# Exterior colour finishes

The dry dash render finish to blockwork will be specified as a white backing coat with Nordic white chips above a Gleneagles Granite grey coloured facing brick basecourse.

# Floors

The ground floor will be of smooth finished concrete. Upper floors will be timber joisted with chipboard flooring.

# Windows and doors

All windows will be double glazed uPVC; in anthracite colour (equivalent to RAL 7016) finish externally, white internally; with satin chrome finish handles.

Separate roof and combination windows will be of timber construction.

Front entrance doors will be of GRP construction in an uPVC frame with a glazed sidescreen; in anthracite colour (equivalent to RAL 7016) finish externally, white colour internally; with satin chrome finish handles and a letter plate.

Rear entrance doors will be of uPVC construction and fully glazed; in anthracite (equivalent to RAL 7016) finish externally, white internally; with satin chrome finish handles.

Sliding patio doors will be of glazed uPVC construction; in anthracite colour (equivalent to RAL 7016) finish externally, white colour internally; with polished chrome finish handles.

Internal doors will be ladder moulded semi-solid core timber, in Timeless painted finish, with any glazed doors being clear single panel. Handles will be of polished chrome finish with satin grips, all other door ironmongery will be of satin or polished chrome silver finish.

# Internal finishings

Skirting boards and door facings will be square-faced and finished with a rounded edge, with all internal woodwork in Timeless painted finish.

Wardrobes will have a hanging rail/s and Timeless painted finish shelving, fitted with silver mirror/frame sliding doors, except in the case of a narrower opening, where a conventional opening timber door will be used.

# Internal decoration

Walls and ceilings will be of Timeless painted finish.

# Staircase

Staircase balustrades will be of solid, Timeless painted plasterboard finish, with a Timeless painted timber handrail.

# Plumbing, central heating and ventilation

A “wet” system using an air source heat pump with thermostatically controlled radiators will be fitted in all properties.

This incorporates a Daikin Altherma 3 R32 Split system comprising a floor-standing indoor unit with integrated domestic hot water tank, an external fan unit and a programmable room thermostat.

A de-centralised mechanical extract ventilation (dMEV) system will be fitted, comprised of continuously running extractor fans within the kitchens and “wet” rooms.

# Electrical installation

Ample white finished power points will be fitted throughout, with a telephone/digital TV/satellite multi-point in the living room and master bedroom. TV points will be wired to a central distribution point and to the roof space with future installation of an aerial/dish and final connection being the homeowner’s responsibility.

Isolation for kitchen appliances may involve the use of a grid switch in a central location.

A shaver point will be fitted in bathrooms and en suites.

All rooms (including Utility cupboards) will be fitted with ceiling light pendants, with a single additional downlighter fitted above shower cubicles. All lighting fixtures will be installed with LED bulbs and positions of electrical points can be seen on plot specific drawings.

Some house designs will also have a mix of feature lighting within the kitchen.

External lighting positions can be seen on plot specific drawings. Public elevations will be fitted with a stainless steel up and downlighter with PIR sensor, with a bulkhead style light fitting on private elevations.

A doorbell will be fitted.

# Fibre broadband internet connectivity

Infrastructure will be installed at the development to provide FTTP (Fibre to the Premise) technology, which provides estimated speeds of up to 300Mbps at the entry point to the home.

Final connection and service provision within the home remains the homeowner’s responsibility and ultimate connection speeds will be dependent on the service provided by the chosen communications provider.

# Kitchen

A choice of cabinets and varied work surfaces are offered as standard, and will include built-in stainless steel electric oven, electric induction hob, recirculating extractor hood and a tall integrated fridge/freezer.

A coordinating laminate up-stand will be fitted above the work surface with a glass splash-back behind the hob.

# Utility rooms & cupboards

Where applicable, utility rooms will be fitted with the same style of cabinets and work surfaces/upstands as chosen for the kitchen.

Designated spaces on kitchen drawings or in Utility cupboards will be fitted with plumbing (cold water feed and drainage) and electrics for connecting purchaser’s own single or dual self-condensing appliances.

# W.C., bathrooms and en suites

All will comprise a w.c. and wash hand basin from a range which is of contemporary appearance with appropriate style tap fitments etc.

Where there is no en suite, the bath in the bathroom will be fitted with a separate thermostatically controlled flexible shower kit and screen.

En suites will have a shower tray with upstand on panelled edges, complete with screen and thermostatically controlled valve.

Fitted mirrors are to be located above basins in bathrooms and en suites, where possible.

Choice of wall tiling to sill height on a minimum of the basin surrounding wall panel in bathrooms and, where appropriate, en suites.

Where an over bath shower is fitted as standard, there will be tiling to a height of approximately 1800mm on the 3 walls above the bath, otherwise one row of tiling around the bath in bathrooms.

Gloss white 8100 wet wall panelling will be fitted within the shower cubicle of an en-suite.

# Gardens

Where possible, gardens will be graded to suit the contours of the development and in some cases this may involve the creation of slopes instead of retaining walls.

Turf will be laid to front gardens and shrubbery may be installed in some areas in accordance with the approved landascaping layout. The rear gardens will be rotovated topsoil with a footpath constructed of grey riven paving slabs leading to a rotary clothes drier, which is provided. Small/isolated areas of rear gardens which are not suitable for future turf, such as between paths and fences/garages etc., may be filled with loose gravel.

Open French drains may be included to aid drainage, these will need to be retained and maintained.

General drainage will be improved by garden establishment and maintenance.

# Parking, driveways & garages

Parking areas and driveways will generally comprise an initial 3000mm of lock block/paviours leading from the public footpath, with the remaining driveway between this and the house being washed gravel held within a cellular paving grid system.

Detached garages will be of single leaf blockwork construction, with exterior wall and roof finishes to complement the associated house.

The vehicular entrance door will be a Garador Carlton retractable door, with vertically lined anthracite (equivalent to RAL 7016) colour finish and a satin finish handle.

The pedestrian entrance door will be an uPVC lined-effect door within an uPVC frame and with brushed satin chrome finish handles.

All garages are equipped with power and lighting.

# Street lighting

Designed in accordance with Local Authority Guidelines.

# Fencing, hedging and walling

Rear gardens will generally be enclosed by 1800mm single-sided hit and miss fencing; changing to shiplap on front/public boundaries; with this height reducing to 900mm on side boundaries between the rear garden and the front of the house, and then changing to hedging running to the front boundary. Feature grey fencing may be used in some areas to satisfy the requirements of Planning approvals.

A mixture of post and wire or 1200mm timber fencing may be used in conjunction with hedging on boundaries that overlook open aspects.

Please refer to site plans and/or ask the Sales Adviser for specific layouts.

Where a boundary is shared with other plots on the development and fencing has previously been erected, this will not be replaced and may be subject to natural weathering.

Installation and growth of hedging may be subject to growing season.

Plot specific variations may be checked on site plans.

# Efflorescence and ‘lime bloom’

Natural salts within wall materials may appear as a white deposit on external and internal walls as part of the normal drying out process. These are neither harmful nor detrimental to wall finish, durability or strength in the long term. Exposure to normal weather conditions and naturally acidic rain water usually removes the majority of such deposits within the first year of completion.

# Factoring

All public open spaces will be maintained by a factor appointed by the developer, please consult the factoring information document for specific details.

# Development Management Scheme and/or Deed of Conditions

All owners will be required to comply with the terms of the Deed of Conditions/Development Management Scheme. For further details please ask the Sales Adviser.

# Warranties

New build residential properties have the benefit of the 10-year NHBC Buildmark cover.

# Applicable Building Warrant

Building (Scotland) Regulations 2004, Technical Handbook 2020 Domestic (effective on all applications from 1st March 2021)

Please read the above in conjunction with site/plot plans and consult your Sales Adviser for further information. Scotia Homes Limited reserves the right to vary the above specifications where necessary but to similar or higher standards.